ITU07209 – VLAN Firewall Lab (Updated)

IT (VLAN 40) & Marketing (VLAN 20) — Cisco Packet Tracer Step-by-Step Guide

Firewall mechanism: Extended ACLs (no Zone-Based Policy Firewall)

# 1. Design Summary

Devices needed:

| Device | Role | Model (any works) |
| --- | --- | --- |
| R1 | Internal router — inter-VLAN routing + ACL firewall | 2911 or 1941 |
| R-ISP | Represents the Internet / ISP | 2911 or 1941 |
| SW1 | Access switch — VLAN 40 & VLAN 20 | 2960 |
| PC1, PC2 | IT workstations | PC |
| Server0 | IT department server | Server |
| PC3, PC4 | Marketing workstations | PC |
| Server1 (optional) | Represents an "Internet" web server, attached to R-ISP | Server |

IP Addressing Plan (updated):

| Segment | Network | Gateway | Notes |
| --- | --- | --- | --- |
| VLAN 40 – IT | 192.168.40.0/24 | 192.168.40.1 | PC1=.10, PC2=.11, Server0=.20 |
| VLAN 20 – Marketing | 192.168.20.0/24 | 192.168.20.1 | PC3=.10, PC4=.11 |
| WAN link R1↔R-ISP | 200.200.200.0/30 | R1=.1 / R-ISP=.2 | Ethernet only — no serial |
| "Internet" server | 8.8.8.0/24 | 8.8.8.1 (on R-ISP) | Server1 = 8.8.8.8 |

Firewall design (Extended ACLs — no ZBF/zone commands):

- MKT-TO-IT-BLOCK — applied inbound on VLAN 20's sub-interface — denies Marketing→IT, permits everything else (so Marketing→Internet still works).

- IT-TO-MKT-BLOCK — applied inbound on VLAN 40's sub-interface — denies IT→Marketing, permits everything else.

This satisfies the same three requirements as a Zone-Based Policy Firewall would, without needing the securityk9 license package (which some Packet Tracer router images do not activate).

Same-VLAN traffic (e.g. PC1↔PC2↔Server0) never touches R1 at all — it's switched locally by SW1, so it's unaffected by any ACL.

# 2. Build the Topology in Packet Tracer

- Open Packet Tracer, place R1, R-ISP, SW1.

- Place PC1, PC2, Server0 and connect to SW1 (copper straight-through) on ports Fa0/1, Fa0/2, Fa0/3.

- Place PC3, PC4 and connect to SW1 on ports Fa0/4, Fa0/5.

- Connect SW1 Gi0/1 → R1 Gi0/0 (trunk / router-on-a-stick link) — copper straight-through cable only.

- Connect R1 Gi0/1 → R-ISP Gi0/0 (WAN link) — copper straight-through cable, not serial.

- (Optional) Place Server1 on R-ISP's side (e.g. R-ISP Gi0/1 → Server1), representing "the internet" — copper straight-through.

- Confirm every cable icon shows solid green dots on both ends before moving on. A dashed cable icon means Packet Tracer picked a serial link by mistake — delete it and reconnect with the copper straight-through tool instead.

# 3. Configure the Switch (SW1)

enable
configure terminal
hostname SW1
 
vlan 40
 name IT
exit
vlan 20
 name MARKETING
exit
 
interface range fa0/1-3
 switchport mode access
 switchport access vlan 40
exit
 
interface range fa0/4-5
 switchport mode access
 switchport access vlan 20
exit
 
interface gi0/1
 switchport mode trunk
 switchport trunk allowed vlan 40,20
exit
 
! Optional but recommended: speeds up port convergence on access ports
interface range fa0/1-5
 spanning-tree portfast
exit
 
end
write memory

# 4. Configure R1 (Router-on-a-Stick + NAT)

enable
configure terminal
hostname R1
 
interface gi0/0
 no shutdown
exit
 
interface gi0/0.40
 encapsulation dot1Q 40
 ip address 192.168.40.1 255.255.255.0
 ip nat inside
exit
 
interface gi0/0.20
 encapsulation dot1Q 20
 ip address 192.168.20.1 255.255.255.0
 ip nat inside
exit
 
interface gi0/1
 ip address 200.200.200.1 255.255.255.252
 ip nat outside
 no shutdown
exit
 
ip route 0.0.0.0 0.0.0.0 200.200.200.2
 
! --- NAT (PAT) so both VLANs can reach the internet ---
access-list 1 permit 192.168.40.0 0.0.0.255
access-list 1 permit 192.168.20.0 0.0.0.255
ip nat inside source list 1 interface gi0/1 overload

# 5. Configure R-ISP (represents the Internet)

enable
configure terminal
hostname R-ISP
 
interface gi0/0
 ip address 200.200.200.2 255.255.255.252
 no shutdown
exit
 
interface gi0/1
 ip address 8.8.8.1 255.255.255.0
 no shutdown
exit
 
ip route 192.168.40.0 255.255.255.0 200.200.200.1
ip route 192.168.20.0 255.255.255.0 200.200.200.1

Configure Server1 (if used): IP 8.8.8.8, mask 255.255.255.0, gateway 8.8.8.1. Enable its HTTP service in Desktop → Services if you want to test a browser page too.

# 6. Configure End Devices

| Device | IP | Mask | Gateway | DNS |
| --- | --- | --- | --- | --- |
| PC1 | 192.168.40.10 | /24 | 192.168.40.1 | 8.8.8.8 |
| PC2 | 192.168.40.11 | /24 | 192.168.40.1 | 8.8.8.8 |
| Server0 | 192.168.40.20 | /24 | 192.168.40.1 | 8.8.8.8 |
| PC3 | 192.168.20.10 | /24 | 192.168.20.1 | 8.8.8.8 |
| PC4 | 192.168.20.11 | /24 | 192.168.20.1 | 8.8.8.8 |

📸 Screenshot #1: IP Configuration window of one PC from each VLAN.

# 7. Configure the Firewall (Extended ACLs — no Zone-Based Policy Firewall)

enable
configure terminal
 
! --- Step A: Create the two ACLs ---
ip access-list extended MKT-TO-IT-BLOCK
 deny ip 192.168.20.0 0.0.0.255 192.168.40.0 0.0.0.255
 permit ip any any
exit
 
ip access-list extended IT-TO-MKT-BLOCK
 deny ip 192.168.40.0 0.0.0.255 192.168.20.0 0.0.0.255
 permit ip any any
exit
 
! --- Step B: Apply each ACL inbound on the matching sub-interface ---
interface gi0/0.20
 ip access-group MKT-TO-IT-BLOCK in
exit
 
interface gi0/0.40
 ip access-group IT-TO-MKT-BLOCK in
exit
 
end
write memory

Why this works: Each ACL's deny line matches only traffic aimed at the other department's subnet; the trailing permit ip any any still allows that same VLAN's traffic to reach the internet (or anywhere else) normally. Because the ACL is applied in on each VLAN's own sub-interface, R1 evaluates it the moment traffic enters from that VLAN — before it can be routed anywhere, including toward the other VLAN.

📸 Screenshot #2: show access-lists (after running your ping tests — match counters will show real numbers).

📸 Screenshot #3: show ip interface gi0/0.40 — confirms Inbound access list is IT-TO-MKT-BLOCK.

📸 Screenshot #4: show ip interface gi0/0.20 — confirms Inbound access list is MKT-TO-IT-BLOCK.

# 8. Testing Plan & Expected Results

Run these from each PC's Command Prompt (Desktop → Command Prompt).

| # | Test | From → To | Expected Result |
| --- | --- | --- | --- |
| 1 | ping 192.168.40.11 | PC1 → PC2 (same VLAN) | ✅ Success |
| 2 | ping 192.168.40.20 | PC1 → Server0 (same VLAN) | ✅ Success |
| 3 | ping 8.8.8.8 | PC1 → Internet | ✅ Success |
| 4 | ping 8.8.8.8 | PC3 (Marketing) → Internet | ✅ Success |
| 5 | ping 192.168.40.20 | PC3 (Marketing) → Server0 (IT) | ❌ Destination host unreachable |
| 6 | ping 192.168.40.10 | PC3/PC4 → PC1/PC2 (IT) | ❌ Destination host unreachable |
| 7 | ping 192.168.20.10 | PC1 (IT) → PC3 (Marketing) | ❌ Destination host unreachable |

📸 Screenshots #5–11: one per test above, showing the Command Prompt result.

# 9. Verification Commands for Your Report

show access-lists
show ip interface gi0/0.40
show ip interface gi0/0.20
show ip nat translations
show running-config

📸 Screenshot #12: show ip nat translations right after test #3 or #4 — shows a real translated entry, proving NAT + internet access work end-to-end.

# 10. Suggested Report Structure (for submission)

- Cover page (name, ID, course code)

- Objective & scenario summary

- Network topology diagram (screenshot of full Packet Tracer canvas)

- IP addressing table (Section 1 above)

- Configuration listings (switch, R1, R-ISP) — paste the commands + show run screenshots

- Firewall policy explanation (the two ACLs, why each line is there) with reasoning

- Test results table with screenshots (Section 8)

- Conclusion — confirm each of the 3 original requirements was met:

- Both VLANs reach internet ✅

- IT can talk to itself, protected from Marketing ✅

- Marketing internet-only, no IT/server access ✅

# Quick Troubleshooting Tips

- If a device can't reach its own gateway: check "no shutdown" was applied on both physical and sub-interfaces.

- If internet ping fails for everyone: check the NAT ACL (access-list 1) includes both subnets, and that ip nat inside/outside are on the correct interfaces — verify with show running-config if the filtered show ip interface | include NAT command returns nothing (this is a known Packet Tracer display quirk, not necessarily a real problem).

- If Marketing unexpectedly reaches IT (or vice versa): confirm each ip access-group is applied inbound (in) on the correct sub-interface — an ACL applied outbound or on the wrong sub-interface won't catch the traffic.

- If two PCs in the same VLAN suddenly can't ping each other: this is a Layer 2 issue, not a router/ACL issue (same-VLAN traffic never reaches R1). Check show mac-address-table on the switch — if a device's MAC address is missing or shows up on the wrong port, re-seat that physical cable in Packet Tracer.

- If a port was just brought up or reconnected, allow it a few seconds to leave Spanning Tree's listening/learning state — show spanning-tree vlan <id> should show FWD, not LIS/LRN/BLK, before traffic will pass. Applying spanning-tree portfast on access ports (Section 3) avoids this delay.

- Router reloads clear any unsaved configuration — always write memory after a working set of changes, and re-verify with show running-config after any reload.